STANDARD OPERATING PROCEDURE AND CHECKLIST S6/S7 Robots

| ITEM | ACTIONS PERFORMED | YES | NO |
| --- | --- | --- | --- |
|  | GENERAL: |  |  |
| 1 | Perform visual inspection of machine | ☐ | ☐ |
| 2 | General check for structural defects | ☐ | ☐ |
| 3 | Tighten all fasteners | ☐ | ☐ |
| 4 | Verify safety functionality | ☐ | ☐ |
|  | S6/S7 Robots |  |  |
| 5 | Walk Robot around a pallet to check correct rotation and bearing wear | ☐ | ☐ |
| 6 | Lift Robot to inspect bottom; remove film build up; check bearings | ☐ | ☐ |
| 7 | Check Motor connection and break connection; check for oil leaks | ☐ | ☐ |
| 8 | check the yellow chain sprocket; clean debris if any is present | ☐ | ☐ |
| 9 | Check drive motor, gearbox oil and drive sprocket alignment | ☐ | ☐ |
|  | FILM CARRIAGE S6/S7 |  |  |
| 10 | Check film carriage for correct operation | ☐ | ☐ |
| 11 | Clean out all debris, and clean and lubricate bearings / wheels | ☐ | ☐ |
| 12 | Check film operation manually, check pre-stretch | ☐ | ☐ |
| 13 | Clean film rollers, and gears, and clutch | ☐ | ☐ |
| 14 | Check gears and belts on the S6 clean and test, S7 check for Oil on the Motor gear box assembly. | ☐ | ☐ |
| 15 | Check all limit switches, photo eyes and encoders for correct operation | ☐ | ☐ |
| 16 | Check and adjust carriage guide wheels and bracket connection | ☐ | ☐ |
|  | MAST S6/S7 |  |  |
| 17 | Check guide shafts and support frame for wear and rigidity | ☐ | ☐ |
| 18 | Check lift chain for correct tension and lubrication | ☐ | ☐ |
| 19 | Check carriage lift motor | ☐ | ☐ |
| 20 | Check carriage lift gearbox for correct operation and oil level | ☐ | ☐ |
| 21 | Check all sprockets for correct alignment and condition | ☐ | ☐ |
|  | ELECTRICAL COMPONENTS S6 /S7 |  |  |
| 22 | Check wiring harness internally, check for condition of individual cables | ☐ | ☐ |
| 23 | Check all cable connections for fraying and abrasions | ☐ | ☐ |
| 24 | Check control panel for loose or bad connections | ☐ | ☐ |
| 25 | Inspect wiring harness internally to check condition of individual wires | ☐ | ☐ |
| 26 | Check all buttons and switches for correct operation and wear | ☐ | ☐ |
| 27 | Check for signs of burning wires | ☐ | ☐ |
| 28 | Check all indicator lights | ☐ | ☐ |
| 29 | Check grounding for continuity | ☐ | ☐ |
|  | TEST OF THE EQUIPMENT: |  |  |
| 30 | Perform cycle testing on at least 5 pallets | ☐ | ☐ |